# Real-Time GPS Vehicle Tracking System using STM32F103C6T6A, ESP32 and Local Web Server

# Abstract

This project implements a real-time GPS vehicle tracking system using an STM32F103C6T6A microcontroller, a NEO-6M GPS module, and an ESP32 Wi-Fi module. The STM32 receives GPS data, forwards it to the ESP32 through UART, and the ESP32 uploads the information to a local web server over Wi-Fi. A browser dashboard displays the vehicle location, speed, date, time, and GPS status in real time.

# Objectives

- Acquire real-time GPS location.
- Interface GPS with STM32 using UART.
- Transfer GPS data to ESP32.
- Send data to a local web server.
- Display live information on a web dashboard.

# Hardware Components

• STM32F103C6T6A
• NEO-6M GPS Module
• ESP32 DevKit
• Wi-Fi Router
• PC/Laptop (Local Server)
• USB-TTL Programmer
• 3.3V/5V Power Supply

# Software Used

Keil uVision 5
STM32 HAL Library
Arduino IDE (ESP32)
HTML/CSS/JavaScript
XAMPP or Python Flask

# Working Principle

1. GPS module receives satellite signals.
2. GPS sends NMEA sentences to STM32 via UART1.
3. STM32 forwards the GPS data to ESP32 through UART2.
4. ESP32 connects to the Wi-Fi network.
5. ESP32 sends GPS data to the local server using HTTP.
6. The local server stores and displays the data on a web dashboard.

# Pin Connections

GPS TX -> STM32 USART1 RX
GPS RX -> STM32 USART1 TX (optional)
STM32 USART2 TX -> ESP32 RX
STM32 USART2 RX -> ESP32 TX
Common GND between all devices

# Advantages

• Low-cost implementation
• Real-time tracking
• No GSM charges within local Wi-Fi
• Easy to expand
• Reliable UART communication

# Applications

Vehicle tracking
College/School buses
Fleet management
Logistics
Asset tracking
Personal safety

# Future Scope

Cloud integration
Google Maps live tracking
Mobile app
Geo-fencing
Database analytics
SMS/Email alerts

# Conclusion

The project demonstrates a complete embedded IoT solution by integrating GPS, STM32, ESP32, and a local web server. It provides accurate and real-time vehicle location monitoring while offering a scalable architecture for future enhancements.

# Block Diagram